Page 1of 4

SAFETY DATA SHEET

Issue Date :March 2014

BEESWAX FURNITURE POLISH

CLASSIFIED AS HAZARDOUS ACCORDING TO THE CRITERIA OF WORKSAFE AUSTRALIA

- PRODUCT AND SUPPLIER IDENTIFICATION

Trade name  		Beeswax Furniture Polish

Product class		Wax and turpentine polish

Intended use	Furniture wax and polish

Company name 	Murobond Coatings Pty Ltd

81 – 83 Dickson Ave,

ARTARMON NSW 2064

Telephone no.		02 9906-7299

Fax no.			02 9439-7593

- HAZARD IDENTIFICATION

Classification		Flammable Liquids: Category 3

Hazard statement	Flammable gel and vapour.

Harmful in contact with skin.

May cause sensitisation of skin.

Harmful if inhaled.

May cause lung damage if swallowed

Toxic to aquatic organisms, may cause long term adverse effects.

Prevention		Do not breathe mist/vapours/spray.

statement 		Do not get in eyes, on skin, or on clothing.

Keep away from heat/sparks/open flames/hot surfaces. . No smoking. Keep container tightly closed.

Wear protective gloves/protective clothing/eye protection/face protection. Use only outdoors or in a well-ventilated area.

Poison schedule	S5

Storage statement	Keep out of reach of children. Store in secure, cool, dry place.

Keep containers tightly closed, away from heat, sources of ignition, and strong oxidising agents. Prevent from freezing.

Disposal statement	Dispose of in compliance with all government regulations.

- COMPOSITION / INFORMATION ON INGREDIENTS

Composition		Beeswax and gum turpentine

Ingredients 	Name 					CAS 		Proportion

Yellow bees wax	 				30-60 %

Gum turpentine				8006-64-2	30-60 %

- FIRST AID MEASURES

Inhalation	No toxic vapours when working in a well ventilated area. Remove victim to fresh air and keep at rest in a position comfortable for breathing. Administer artificial respiration if breathing is stopped. Keep at rest. Seek medical attention.

Ingestion	DO NOT induce vomiting. Wash mouth and lips with copious amounts of water and give plenty to drink. Seek medical advice.

Page 2 of 4 Murobond Beeswax Furniture Polish

Skin	If skin or hair contact occurs, remove contaminated clothing. Wash contact areas with soap and water. Wash contaminated clothing before re-use.

Eye contact	If in eyes, hold eyelids apart and flush continuously with running water. Continue flushing for at least 15 minutes. Seek medical attention if irritation persists.

Advice to Doctors	Treat symptomatically.

- FIRE FIGHTING MEASURES

General hazards:	Use water to cool fire-exposed containers. If a leak or spill has not

ignited, use water spray to disperse the vapours and to protect personnel attempting to stop leak. Water spray may be used to flush spills away from exposures. Prevent runoff from fire control or dilution from entering waterways, sewers or drinking water supply.

Fire fighting extinguishing media: Use sand , foam, dry chemical  powder or carbon

dioxide.

Fire fighting equipment: Fire fighters to wear protective clothing and self-contained

breathing apparatus if risk of exposure to vapour or products of

combustion /decomposition.

Hazards from combustion products: Carbon monoxide, carbon dioxide.

- ACCIDENTAL RELEASE

Spills & disposal	Eliminate all sources of ignition. Remove leaking containers to

detached area. Wear protective equipment to prevent skin and eye

contamination and inhalation of vapours. Contain - prevent

contamination of drains and waterways. Use absorbent (soil, sand,

vermiculite). Collect and seal in properly labelled drums for

disposal. Prevent spills from contacting soil, entering waterways or stormwater drains.

- HANDLING AND STORAGE

Safe handling	Keep out of reach of children. Avoid contact with skin. Use in well

ventilated area away from all ignition sources. Wear protective clothing to prevent skin and eye contact.

Safe storage 	Store in tightly closed containers in a cool area away from all ignition sources and strong oxidising agents. Prevent from freezing.

- EXPOSURE CONTROLS/PERSONAL PROTECTION

Engineering controls 	Use in a well ventilated area - keep containers closed when not in

use. Exhaust ventilation may be required for poorly ventilated work areas or when product is heated (refer AS1668).

Respiratory protection If risk of inhalation of vapour exists, wear a combined organic

vapour / particulate respirator complying with AS/NZS 1715/1716.

Eye protection		Safety glasses or goggles (Refer AS 1337).

Hand Protection	Laminated film, butyl reusable gloves. (refer AS 2161.1)

Footwear 		Enclosed footwear.

Body Protection 	Overalls or similar protective apparel.

Hygiene Measures 	Wash hands and face before smoking, eating, drinking or using the

toilet.

Page 3 of 4 Murobond Beeswax Furniture Polish

- PHYSICAL AND CHEMICAL PROPERTIES

Appearance 		Off white to yellow coloured wax.

Odour 			Turpentine odour.

Boiling point 		160°C. Approximately

Solubility in water 	Negligible.

Specific gravity 	Approx. 0.96 @ 20 Deg C.

Flash point 		32 C. TCC

Auto-ignition temp.	 >200 Deg C.

Flammable limits lower: 0.7%

Flammable limits upper: 5.6%

- STABILITY AND REACTIVITY

Chemical stability: 	Stable under normal usage conditions

Conditions to avoid:	Avoid contact with food, heat, sparks, flame and build up of static electricity.

Incompatible materials: Strong oxidisers.

Hazardous decomposition products: Carbon monoxide.

Hazardous reactions: 	None known.

- TOXICOLOGICAL INFORMATION

No toxicological date is available for this product. No adverse health effects if product is handled in accordance with this Safety Data Sheet. Health effects that may occur if this product is mishandled are:

Inhalation	No toxic vapour. If this product is used in a poorly ventilated area, vapour may cause irritation to mucous membranes, nausea, or headaches.

Skin contact		May cause irritation to people with sensitive skin.

Eye contact		May be an eye irritant.

Ingestion	Harmful. May cause gastro-intestinal irritation and haemorrhage and congestion of intestines.

- ECOLOGICAL INFORMATION

Ecotoxicity		Prevent product entering stormwater system and waterways

- DISPOSAL CONSIDERATIONS

Waste disposal	Comply with local, state and federal regulations. Do not dispose into sewerage. Leave empty containers open in a well ventilated area to dry. The product issuitable for processing by an approved recycling facility or can be disposed of at any licensed waste disposal site.

- TRANSPORT INFORMATION

U.N. Number			1299

UN proper shipping name 	TURPENTINE

Transport hazard class 	3

Hazchem Code 		3Y

Packing Group 		III

D.G. Class			3.2

- REGULATORY INFORMATION

Risk phrases

R 10			Flammable

R 20/21/22		Harmful by inhalation, in contact with skin and if swallowed

R 36/38 		Irritating to eyes and skin

Page 4 of 4 Murobond Beeswax Furniture Polish

R 43		 	May cause sensitisation of skin with repeated contact

R 65			Harmful, may cause lung damage if swallowed

R 51/53	Toxic to aquatic organisms. May cause long-term adverse effects to aquatic environments

Safety phrases

S 2	Keep out of reach of children

S 36/37 	Wear suitable protective clothing and gloves

S 61	Avoid release to the environment

S 62	If swallowed, do not induce vomiting, seek medical advice immediately.

Poisons Schedule	S5

Hazard Category	Xn Harmful, F Flammable, N Dangerous to the environment

- OTHER INFORMATION

Contact person/point	info@murobond.com.au

Manager, Technical Services

Phone: 02 9906 7299

Fax: 02 9439 7593

E:mail: info@murobond.com.au

Reason issued	New SDS to comply with the Code of Practice for Preparation of Safety Data Sheets for Hazardous Substances (December 2011) and Labelling of Workplace Hazardous Chemicals (August 2011).